#include <WiFi.h>

#include <WebServer.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include "HX711.h"

#include <RTClib.h>

#include <Preferences.h>

// ===== WIFI SETTINGS =====

const char* ssid = "MediTrack_Fast";

const char* password = "12345678";

WebServer server(80);

Preferences prefs;

// ===== PIN DEFINITIONS =====

#define DT1 4

#define SCK1 5

#define DT2 26

#define SCK2 27

#define BUZZER 23

#define LED_PIN 2

#define BUTTON 25

HX711 scale1, scale2;

LiquidCrystal_I2C lcd(0x27, 16, 2);

RTC_DS3231 rtc;

// ===== VARIABLES =====

float weight1 = 0, weight2 = 0;

float smooth1 = 0, smooth2 = 0;

float gppMorning = 2.0, gppEvening = 2.0;

float cal1 = 290.0, cal2 = 290.0;

int mHour = 8, mMin = 30;

int eHour = 20, eMin = 30;

bool alarmActive = false;

int lastAlarmMinute = -1;

unsigned long lastBeep = 0;

bool buzState = false;

bool stockAlertDone = false;

// ================= LOAD/SAVE =================

void loadSettings() {

prefs.begin("med", true);

gppMorning = prefs.getFloat("gppm", 2.0);

gppEvening = prefs.getFloat("gppe", 2.0);

mHour = prefs.getInt("mh", 8);

mMin = prefs.getInt("mm", 30);

eHour = prefs.getInt("eh", 20);

eMin = prefs.getInt("em", 30);

prefs.end();

}

void saveSettings() {

prefs.begin("med", false);

prefs.putFloat("gppm", gppMorning);

prefs.putFloat("gppe", gppEvening);

prefs.putInt("mh", mHour);

prefs.putInt("mm", mMin);

prefs.putInt("eh", eHour);

prefs.putInt("em", eMin);

prefs.end();

}

// ================= WEB DASHBOARD =================

String webpage() {

return R"====(

<!DOCTYPE html>

<html>

<head>

<meta name="viewport" content="width=device-width, initial-scale=1">

<title>MediTrack FAST</title>

<style>

body { font-family: sans-serif; background: #f1f5f9; text-align: center; padding: 20px; }

.card { background: white; padding: 20px; border-radius: 10px; box-shadow: 0 2px 5px rgba(0,0,0,0.1); margin: 10px auto; max-width: 400px; }

.val { font-size: 24px; font-weight: bold; color: #2563eb; }

button { background: #2563eb; color: white; border: none; padding: 10px 20px; border-radius: 5px; cursor: pointer; width: 100%; margin-top:10px; }

input { padding: 5px; width: 40px; text-align: center; }

</style>

<script>

function update(){

fetch('/data').then(r=>r.json()).then(d=>{

document.getElementById("t").innerHTML = d.time;

document.getElementById("m").innerHTML = d.mw + "g (Qty-" + d.mq + ")";

document.getElementById("e").innerHTML = d.ew + "g (Qty-" + d.eq + ")";

});

}

setInterval(update, 500);

</script>

</head>

<body onload="update()">

<div class="card"><h2>MediTrack</h2><p id="t">00:00:00</p></div>

<div class="card">Morning: <span class="val" id="m">0</span></div>

<div class="card">Evening: <span class="val" id="e">0</span></div>

<div class="card">

<form action="/setAlarm">

M: <input name="mh" placeholder="H">:<input name="mm" placeholder="M"><br>

E: <input name="eh" placeholder="H">:<input name="em" placeholder="M"><br>

<button type="submit">Set Schedule</button>

</form>

</div>

</body>

</html>

)====";

}

void handleData() {

DateTime now = rtc.now();

char tBuf[10]; sprintf(tBuf, "%02d:%02d:%02d", now.hour(), now.minute(), now.second());

String json = "{";

json += "\"time\":\"" + String(tBuf) + "\",";

json += "\"mw\":" + String((int)round(weight1)) + ",";

json += "\"ew\":" + String((int)round(weight2)) + ",";

json += "\"mq\":" + String((int)round(weight1/gppMorning)) + ",";

json += "\"eq\":" + String((int)round(weight2/gppEvening)) + "";

json += "}";

server.send(200, "application/json", json);

}

// ================= SETUP =================

void setup() {

Serial.begin(115200);

pinMode(BUZZER, OUTPUT); pinMode(LED_PIN, OUTPUT); pinMode(BUTTON, INPUT_PULLUP);

Wire.begin(21, 22); lcd.init(); lcd.backlight(); rtc.begin();

scale1.begin(DT1, SCK1); scale2.begin(DT2, SCK2);

scale1.set_scale(cal1); scale2.set_scale(cal2);

scale1.tare(); scale2.tare();

loadSettings();

WiFi.softAP(ssid, password);

server.on("/", [](){ server.send(200, "text/html", webpage()); });

server.on("/data", handleData);

server.on("/setAlarm", [](){

if(server.hasArg("mh")) mHour = server.arg("mh").toInt();

if(server.hasArg("mm")) mMin = server.arg("mm").toInt();

if(server.hasArg("eh")) eHour = server.arg("eh").toInt();

if(server.hasArg("em")) eMin = server.arg("em").toInt();

saveSettings(); server.send(200, "text/html", "Saved <a href='/'>Back</a>");

});

server.begin();

}

// ================= LOOP =================

void loop() {

server.handleClient();

// Fast Weight Sensing

if (scale1.is_ready()) {

float r1 = scale1.get_units(1);

smooth1 = (0.2 * r1) + (0.8 * smooth1);

weight1 = (smooth1 < 0.5) ? 0 : smooth1;

}

if (scale2.is_ready()) {

float r2 = scale2.get_units(1);

smooth2 = (0.2 * r2) + (0.8 * smooth2);

weight2 = (smooth2 < 0.5) ? 0 : smooth2;

}

int mq = (int)round(weight1 / gppMorning);

int eq = (int)round(weight2 / gppEvening);

// Low Stock Buzzer (2s alert)

if ((mq < 3 || eq < 3) && (mq > 0 || eq > 0)) {

if (!stockAlertDone) {

digitalWrite(BUZZER, HIGH);

delay(2000);

digitalWrite(BUZZER, LOW);

stockAlertDone = true;

}

} else if (mq >= 3 && eq >= 3) {

stockAlertDone = false;

}

// Alarm Logic

DateTime now = rtc.now();

if ((now.hour() == mHour && now.minute() == mMin) || (now.hour() == eHour && now.minute() == eMin)) {

if (lastAlarmMinute != now.minute()) { alarmActive = true; lastAlarmMinute = now.minute(); }

}

if (digitalRead(BUTTON) == LOW) alarmActive = false;

if (alarmActive) {

digitalWrite(LED_PIN, (millis() / 200) % 2);

if (millis() - lastBeep > 150) {

lastBeep = millis(); buzState = !buzState;

digitalWrite(BUZZER, buzState);

}

} else {

digitalWrite(LED_PIN, LOW);

digitalWrite(BUZZER, LOW);

}

// ===== LCD FIXED DISPLAY (Always showing Clock & Alarms) =====

static unsigned long lastLCD = 0;

static bool showWeightPage = false;

// Toggle pages every 4 seconds

if (millis() - lastLCD > 4000) {

lastLCD = millis();

showWeightPage = !showWeightPage;

lcd.clear();

}

if (!showWeightPage) {

// Page 1: TIME and ALARM SCHEDULE (Jaise pahle tha)

lcd.setCursor(0, 0);

char tBuf[17];

sprintf(tBuf, "Time: %02d:%02d:%02d", now.hour(), now.minute(), now.second());

lcd.print(tBuf);

lcd.setCursor(0, 1);

char sBuf[17];

sprintf(sBuf, "M:%02d:%02d E:%02d:%02d", mHour, mMin, eHour, eMin);

lcd.print(sBuf);

}

else {

// Page 2: QTY and WEIGHT (From your paper note)

lcd.setCursor(0, 0);

char mBuf[17];

sprintf(mBuf, "M:%dg (Qty-%d)", (int)round(weight1), mq);

lcd.print(mBuf);

lcd.setCursor(0, 1);

char eBuf[17];

sprintf(eBuf, "E:%dg (Qty-%d)", (int)round(weight2), eq);

lcd.print(eBuf);

}

}